SCOPE OF WORK FOR CONVEYOR BELT INSTALLATION, SPLICING AND PULLEY LAGGING SERVICES

ANNEXURE “A”

SCHEDULE OF RATES AND PRICES FOR SERVICES

### Equipment rates

| Equipment rates | Rate per Activity (Excluding VAT) |
| --- | --- |
| Generator 220V |  |
| Generator 380V |  |
| ### Travel Rate for Vehicle Normal, Overtime, Public Holidays/Sunday Rates per team of 5; Rate per Kilometre (Excluding VAT). Travel (Fuel) rate for Vehicle. ### Rates – Normal time, Overtime, weekly and Saturdays (1.5) and Overtime, Sunday/Public holiday (2.0) |  |
| Normal, Overtime, Public Holidays/Sunday Rates per team of 5 | Rate per Hour (VAT Exclusive) |
| Normal Hours |  |
| Overtime Hours (1.5) |  |
| Public Holidays/Sunday hours (2.0) |  |

### Stand By Crew – Parsons, Load out, King and Bruce mines.

| Standby Crew and Equipment – (All Inclusive rate, i.e. transport, tools, PPE etc.) | Description | Quantity | Rate per hour (Excluding VAT) | Total monthly hours | Total Monthly Rate (Excluding VAT) | Unit of Measure |
| --- | --- | --- | --- | --- | --- | --- |
|  | Supervisor | 1 |  | 185 |  | Monthly |
|  | Splicer | 1 |  | 185 |  | Monthly |
|  | Semi-Skilled | 2 |  | 185 |  | Monthly |
|  | Skilled Assistant | 1 |  | 185 |  | Monthly |
|  | Total monthly rate for parsons Standby crew (Excluding VAT) |  |  |  |  |  |

### Stand By Crew – Load-out

| Standby Crew and Equipment – (All Inclusive rate, i.e. transport, tools, PPE etc.) | Description | Quantity | Rate per hour (Excluding VAT) | Total monthly hours | Total Monthly Rate (Excluding VAT) | Unit of Measure |
| --- | --- | --- | --- | --- | --- | --- |
|  | Supervisor | 1 |  | 185 |  | Monthly |
|  | Splicer | 1 |  | 185 |  | Monthly |
|  | Semi-Skilled | 2 |  | 185 |  | Monthly |
|  | Skilled Assistant | 1 |  | 185 |  | Monthly |
|  | Total monthly rate for Load-out Standby crew (Excluding VAT) |  |  |  |  |  |

### Stand By Crew – Bruce & King

| Standby Crew and Equipment – (All Inclusive rate, i.e. transport, tools, PPE etc.) | Description | Quantity | Rate per hour (Excluding VAT) | Total monthly hours | Total Monthly Rate (Excluding VAT) | Unit of Measure |
| --- | --- | --- | --- | --- | --- | --- |
|  | Supervisor | 1 |  | 185 |  | Monthly |
|  | Splicer | 1 |  | 185 |  | Monthly |
|  | Semi-Skilled | 2 |  | 185 |  | Monthly |
|  | Skilled Assistant | 1 |  | 185 |  | Monthly |
|  | Total monthly rate for Bruce & King Standby crew (Excluding VAT) |  |  |  |  |  |

| ### Conveyor Detail: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type | Width (mm) | Class | Ply | Cover | Quantity | Hot Splice - New Installation Per Roll Rate Per Splice (VAT Exclusive) | Hot Splice (Only) & Inserts Rate Per Splice (VAT Exclusive) |
| Plylon | 600 | 630 | 3 | 6/3 | 14 |  |  |
| Plylon | 600 | 1000 | 3 | 6/3 | 1 |  |  |
| Plylon | 750 | 630 | 3 | 6/3 | 6 |  |  |
| Plylon | 750 | 800 | 3 | 6/3 | 2 |  |  |
| Plylon | 750 | 1000 | 3 | 6/3 | 1 |  |  |
| Plylon | 900 | 630 | 3 | 6/3 | 5 |  |  |
| Plylon | 900 | 1250 | 3 | 6/3 | 1 |  |  |
| Plylon | 1050 | 630 | 3 | 6/3 | 3 |  |  |
| Plylon | 1050 | 630 | 3 | 10/4 | 3 |  |  |
| Plylon | 1050 | 800 | 3 | 6/3 | 1 |  |  |
| Plylon | 1050 | 1000 | 3 | 6/3 | 3 |  |  |
| Plylon | 1050 | 1250 | 3 | 6/3 | 7 |  |  |
| Plylon | 1050 | 1600 | 4 | 6/3 | 5 |  |  |
| Plylon | 1200 | 630 | 3 | 10/4 | 6 |  |  |
| Plylon | 1200 | 1600 | 4 | 6/3 | 1 |  |  |
| Plylon | 1350 | 630 | 3 | 10/4 | 2 |  |  |
| Plylon | 1350 | 1250 | 3 | 6/3 | 4 |  |  |
| Plylon | 1350 | 2000 | 4 | 6/3 | 1 |  |  |
| Plylon | 1800 | 2000 | 4 | 6/3 | 1 |  |  |
| Steel Cord | 1050 | 2500 | N/A | 10/4 | 1 |  |  |
| Steel Cord | 1200 | 1600 | N/A | 10/4 | 2 |  |  |
| Steel Cord | 1500 | 1600 | N/A | 10/4 | 4 |  |  |
| Steel Cord | 1500 | 2000 | N/A | 10/4 | 1 |  |  |
| Steel Cord | 1800 | 2000 | N/A | 10/4 | 1 |  |  |
| 1. New installation price will include loop/spool of rolls and pull-in |  |  |  |  |  |  |  |
| 2. Inserts price will include pull-in of insert up to length of 20 meters, should the length of the insert exceeds 20 meter it will be regards as a new installation |  |  |  |  |  |  |  |

| ### Conveyor Pulley Detail |  |  |  |  |
| --- | --- | --- | --- | --- |
| Shore degree Grip 60 x 10mm Diamond pattern lagging for pulleys up to face width of 900 mm |  |  |  |  |
| Shore degree Grip 70 x 12mm Diamond pattern lagging for pulleys wider than 900mm |  |  |  |  |
| Shell Diameter | Shell Face | Quantity | Price - In Position Rate (VAT Exclusive) | Price – Out of position (VAT Exclusive |
| 400 | 700 | 16 |  |  |
| 400 | 900 | 19 |  |  |
| 400 | 1050 | 6 |  |  |
| 400 | 1200 | 2 |  |  |
| 400 | 1350 | 1 |  |  |
| 500 | 700 | 24 |  |  |
| 500 | 900 | 17 |  |  |
| 500 | 1050 | 17 |  |  |
| 500 | 1200 | 27 |  |  |
| 500 | 1350 | 6 |  |  |
| 500 | 1500 | 6 |  |  |
| 630 | 700 | 9 |  |  |
| 630 | 900 | 11 |  |  |
| 630 | 1050 | 9 |  |  |
| 630 | 1200 | 47 |  |  |
| 630 | 1350 | 6 |  |  |
| 630 | 1500 | 20 |  |  |
| 630 | 1700 | 3 |  |  |
| 800 | 700 | 2 |  |  |
| 800 | 900 | 8 |  |  |
| 800 | 1050 | 4 |  |  |
| 800 | 1200 | 52 |  |  |
| 800 | 1350 | 10 |  |  |
| 800 | 1500 | 17 |  |  |
| 800 | 1700 | 13 |  |  |
| 800 | 2000 | 1 |  |  |
| 1000 | 1200 | 31 |  |  |
| 1000 | 1350 | 10 |  |  |
| 1000 | 1500 | 5 |  |  |
| 1000 | 1700 | 15 |  |  |
| 1000 | 2000 | 6 |  |  |
| 1250 | 1500 | 5 |  |  |
| 1250 | 2000 | 5 |  |  |

| ### Conveyor Pulley Detail |  |  |  |  |
| --- | --- | --- | --- | --- |
| Embedded ceramic lagging – Type Nilos, strip lagging, 12mm thick, 4 piece diamond pattern, bonding layer, rubber shore hardness = 50+5, tiles – min. of 92% Aluminium Oxide with minimum tensile strength of 20Mpa Embedded Only on drive applications. |  |  |  |  |
| Shell Diameter | Shell Face | Quantity | Price - In Position | Price - Out of Position |
| 400 | 700 |  |  |  |
| 400 | 900 |  |  |  |
| 400 | 1050 |  |  |  |
| 400 | 1200 |  |  |  |
| 400 | 1350 |  |  |  |
| 500 | 700 |  |  |  |
| 500 | 900 |  |  |  |
| 500 | 1050 |  |  |  |
| 500 | 1200 |  |  |  |
| 500 | 1350 |  |  |  |
| 500 | 1500 |  |  |  |
| 630 | 700 |  |  |  |
| 630 | 900 |  |  |  |
| 630 | 1050 |  |  |  |
| 630 | 1200 |  |  |  |
| 630 | 1350 |  |  |  |
| 630 | 1500 |  |  |  |
| 630 | 1700 |  |  |  |
| 800 | 700 |  |  |  |
| 800 | 900 |  |  |  |
| 800 | 1050 |  |  |  |
| 800 | 1200 |  |  |  |
| 800 | 1350 |  |  |  |
| 800 | 1500 |  |  |  |
| 800 | 1700 |  |  |  |
| 800 | 2000 |  |  |  |
| 1000 | 1200 |  |  |  |
| 1000 | 1350 |  |  |  |
| 1000 | 1500 |  |  |  |
| 1000 | 1700 |  |  |  |
| 1000 | 2000 |  |  |  |
| 1250 | 1500 |  |  |  |
| 1250 | 2000 |  |  |  |

| ### Hot and cold repairs on fabric and steel cord belting – all inclusive |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | Width (mm) | Class | Ply | Cover | Quantity | Normal time - Rate per hour (VAT Exclusive) | Overtime (1.5) - Rate per hour (Weekdays & Saturdays) (VAT Exclusive) | Overtime (2.0) - Rate per hour (Sunday & Public Holiday) (VAT Exclusive) |
| Plylon | 600 | 630 | 3 | 6/3 | 14 |  |  |  |
| Plylon | 600 | 1000 | 3 | 6/3 | 1 |  |  |  |
| Plylon | 750 | 630 | 3 | 6/3 | 6 |  |  |  |
| Plylon | 750 | 800 | 3 | 6/3 | 2 |  |  |  |
| Plylon | 750 | 1000 | 3 | 6/3 | 1 |  |  |  |
| Plylon | 900 | 630 | 3 | 6/3 | 5 |  |  |  |
| Plylon | 900 | 1250 | 3 | 6/3 | 1 |  |  |  |
| Plylon | 1050 | 630 | 3 | 6/3 | 3 |  |  |  |
| Plylon | 1050 | 630 | 3 | 10/4 | 3 |  |  |  |
| Plylon | 1050 | 800 | 3 | 6/3 | 1 |  |  |  |
| Plylon | 1050 | 1000 | 3 | 6/3 | 3 |  |  |  |
| Plylon | 1050 | 1250 | 3 | 6/3 | 7 |  |  |  |
| Plylon | 1050 | 1600 | 4 | 6/3 | 5 |  |  |  |
| Plylon | 1200 | 630 | 3 | 10/4 | 6 |  |  |  |
| Plylon | 1200 | 1600 | 4 | 6/3 | 1 |  |  |  |
| Plylon | 1350 | 630 | 3 | 10/4 | 2 |  |  |  |
| Plylon | 1350 | 1250 | 3 | 6/3 | 4 |  |  |  |
| Plylon | 1350 | 2000 | 4 | 6/3 | 1 |  |  |  |
| Plylon | 1800 | 2000 | 4 | 6/3 | 1 |  |  |  |
| Steel Cord | 1050 | 2500 | N/A | 10/4 | 1 |  |  |  |
| Steel Cord | 1200 | 1600 | N/A | 10/4 | 2 |  |  |  |
| Steel Cord | 1500 | 1600 | N/A | 10/4 | 4 |  |  |  |
| Steel Cord | 1500 | 2000 | N/A | 10/4 | 1 |  |  |  |
| Steel Cord | 1800 | 2000 | N/A | 10/4 | 1 |  |  |  |
| - The above rates will exclude material cost. Material cost will be calculated using the Contractor’s job card and the prices stipulated in the table below titled “Material Cost”. ### Material Prices Part number; Description; Unit Price (Excluding VAT); Unit of measure. 20628481; Repair Strip Fabric – 150mm; Meter. 20628534; Flexco Solid Plate Fastener 21/4 E; Each. 20628654; C16 Grinding Disc; Each. 20628671; Application Brush 75mm; Each. 20628712; Cold Vulcanising Cement Green TL-T60 - set; Set. 20628720; Repair Strip Fabric - 300mm; Meter. 20628848; Repair Strip Fabric – 220mm; Meter. 20647933; Primer Fabric; Litre. 20784329; Cleaning Solvent – TCE Free; Litre. 20790001; EasyFix Conveyor Repair Kit - Non Sag; Set. |  |  |
| #### Should a press be needed for the repair, it will be regarded as a splice |  |  |  |  |  |  |  |  |

| - The Employer (service provider) will provide for 3 x belt presses and 3 x generators in case of abnormal workload, lack of press availabilities, lack of generator availabilities and breakdowns. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | Width (mm) |  | Class |  |  |  |  |  | Ply |  | Cover |  |  |  |  | Quantity |  |  | Rate per Hot Splice – (VAT Exclusive) |  |  |  |
| Plylon | 600 |  | 630 |  |  |  |  |  | 3 |  | 6/3 |  |  |  |  | 14 |  |  |  |  |  |  |
| Plylon | 600 |  | 1000 |  |  |  |  |  | 3 |  | 6/3 |  |  |  |  | 1 |  |  |  |  |  |  |
| Plylon | 750 |  | 630 |  |  |  |  |  | 3 |  | 6/3 |  |  |  |  | 6 |  |  |  |  |  |  |
| Plylon | 750 |  | 800 |  |  |  |  |  | 3 |  | 6/3 |  |  |  |  | 2 |  |  |  |  |  |  |
| Plylon | 750 |  | 1000 |  |  |  |  |  | 3 |  | 6/3 |  |  |  |  | 1 |  |  |  |  |  |  |
| Plylon | 900 |  | 630 |  |  |  |  |  | 3 |  | 6/3 |  |  |  |  | 5 |  |  |  |  |  |  |
| Plylon | 900 |  | 1250 |  |  |  |  |  | 3 |  | 6/3 |  |  |  |  | 1 |  |  |  |  |  |  |
| Plylon | 1050 |  | 630 |  |  |  |  |  | 3 |  | 6/3 |  |  |  |  | 3 |  |  |  |  |  |  |
| Plylon | 1050 |  | 630 |  |  |  |  |  | 3 |  | 10/4 |  |  |  |  | 3 |  |  |  |  |  |  |
| Plylon | 1050 |  | 800 |  |  |  |  |  | 3 |  | 6/3 |  |  |  |  | 1 |  |  |  |  |  |  |
| Plylon | 1050 |  | 1000 |  |  |  |  |  | 3 |  | 6/3 |  |  |  |  | 3 |  |  |  |  |  |  |
| Plylon | 1050 |  | 1250 |  |  |  |  |  | 3 |  | 6/3 |  |  |  |  | 7 |  |  |  |  |  |  |  |  |
| Plylon | 1050 |  | 1600 |  |  |  |  |  | 4 |  | 6/3 |  |  |  |  | 5 |  |  |  |  |  |  |  |  |
| Plylon | 1200 |  | 630 |  |  |  |  |  | 3 |  | 10/4 |  |  |  |  | 6 |  |  |  |  |  |  |  |  |
| Plylon | 1200 |  | 1600 |  |  |  |  |  | 4 |  | 6/3 |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| Plylon | 1350 |  | 630 |  |  |  |  |  | 3 |  | 10/4 |  |  |  |  | 2 |  |  |  |  |  |  |  |  |
| Plylon | 1350 |  | 1250 |  |  |  |  |  | 3 |  | 6/3 |  |  |  |  | 4 |  |  |  |  |  |  |  |  |
| Plylon | 1350 |  | 2000 |  |  |  |  |  | 4 |  | 6/3 |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| Plylon | 1800 |  | 2000 |  |  |  |  |  | 4 |  | 6/3 |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| Steel Cord | 1050 |  | 2500 |  |  |  |  |  | N/A |  | 10/4 |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| Steel Cord | 1200 |  | 1600 |  |  |  |  |  | N/A |  | 10/4 |  |  |  |  | 2 |  |  |  |  |  |  |  |  |
| Steel Cord | 1500 |  | 1600 |  |  |  |  |  | N/A |  | 10/4 |  |  |  |  | 4 |  |  |  |  |  |  |  |  |
| Steel Cord | 1500 |  | 2000 |  |  |  |  |  | N/A |  | 10/4 |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| Steel Cord | 1800 |  | 2000 |  |  |  |  |  | N/A |  | 10/4 |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| Additional Information: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Time Frame for planning, control and benchmarking: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Schedule 1: For splicing only: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Steel cord and Hot splice – Time duration per splice per task |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | Width (mm) |  | Class |  |  |  | Splice Length (mm) |  |  |  |  |  | Total Hours - Splice Only |  |  |  | Total Hours - Splice With Clamping And Pulling Ends – Take-Up Already Lifted |  |  |  |  |  |  |  |
| Plylon | 600 |  | 630 |  |  |  | 1000 |  |  |  |  |  | 4 |  |  |  | 6 |  |  |  |  |  |  |  |
| Plylon | 600 |  | 1000 |  |  |  | 1200 |  |  |  |  |  | 4 |  |  |  | 6 |  |  |  |  |  |  |  |
| Plylon | 750 |  | 630 |  |  |  | 1000 |  |  |  |  |  | 4 |  |  |  | 6 |  |  |  |  |  |  |  |
| Plylon | 750 |  | 800 |  |  |  | 1000 |  |  |  |  |  | 4 |  |  |  | 6 |  |  |  |  |  |  |  |
| Plylon | 750 |  | 1000 |  |  |  | 1200 |  |  |  |  |  | 4 |  |  |  | 6 |  |  |  |  |  |  |  |
| Plylon | 900 |  | 630 |  |  |  | 1000 |  |  |  |  |  | 4 |  |  |  | 6 |  |  |  |  |  |  |  |
| Plylon | 900 |  | 1250 |  |  |  | 1200 |  |  |  |  |  | 4 |  |  |  | 6 |  |  |  |  |  |  |  |
| Plylon | 1050 |  | 630 |  |  |  | 1000 |  |  |  |  |  | 4 |  |  |  | 6 |  |  |  |  |  |  |  |
| Plylon | 1050 |  | 630 |  |  |  | 1000 |  |  |  |  |  | 4 |  |  |  | 6 |  |  |  |  |  |  |  |
| Plylon | 1050 |  | 800 |  |  |  | 1000 |  |  |  |  |  | 4 |  |  |  | 6 |  |  |  |  |  |  |  |
| Plylon | 1050 |  | 1000 |  |  |  | 1200 |  |  |  |  |  | 4 |  |  |  | 6 |  |  |  |  |  |  |  |
| Plylon | 1050 |  | 1250 |  |  |  | 1200 |  |  |  |  |  | 4 |  |  |  | 6 |  |  |  |  |  |  |  |
| Plylon | 1050 |  | 1600 |  |  |  | 1200 |  |  |  |  |  | 4 |  |  |  | 6 |  |  |  |  |  |  |  |
| Plylon | 1200 |  | 630 |  |  |  | 1000 |  |  |  |  |  | 4 |  |  |  | 6 |  |  |  |  |  |  |  |
| Plylon | 1200 |  | 1600 |  |  |  | 1200 |  |  |  |  |  | 4 |  |  |  | 6 |  |  |  |  |  |  |  |
| Plylon | 1350 |  | 630 |  |  |  | 1000 |  |  |  |  |  | 6 |  |  |  | 8 |  |  |  |  |  |  |  |
| Plylon | 1350 |  | 1250 |  |  |  | 1200 |  |  |  |  |  | 6 |  |  |  | 8 |  |  |  |  |  |  |  |
| Plylon | 1350 |  | 2000 |  |  |  | 1500 |  |  |  |  |  | 6 |  |  |  | 8 |  |  |  |  |  |  |  |
| Plylon | 1800 |  | 2000 |  |  |  | 1500 |  |  |  |  |  | 6 |  |  |  | 8 |  |  |  |  |  |  |  |
| Steel Cord | 1050 |  | ST2500 |  |  |  | 1500 |  |  |  |  |  | 6 |  |  |  | 8 |  |  |  |  |  |  |  |
| Steel Cord | 1200 |  | ST1600 |  |  |  | 1200 |  |  |  |  |  | 6 |  |  |  | 8 |  |  |  |  |  |  |  |
| Steel Cord | 1500 |  | ST1600 |  |  |  | 1000 |  |  |  |  |  | 8 |  |  |  | 10 |  |  |  |  |  |  |  |
| Steel Cord | 1500 |  | ST2000 |  |  |  | 1200 |  |  |  |  |  | 8 |  |  |  | 10 |  |  |  |  |  |  |  |
| Steel Cord | 1800 |  | ST2000 |  |  |  | 1600 |  |  |  |  |  | 10 |  |  |  | 12 |  |  |  |  |  |  |  |
| Schedule 2: For New Installation: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Steel cord and Hot splice - Time duration per splice per task |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  | Width(mm) |  |  | Class |  |  | Splice Length (mm) |  |  |  |  |  |  |  |  | Total Hours - Splice Hours with pull-in of belt, Including clamping and overlap of ends, take-up lifted or de-tensioned, pull-in vehicle, and mobile crane assistance |  |  |  |  | Total Hours - Splice Hours with-out pull-in facilities – Including clamping and overlap of ends, take-up lifted or de-tensioned, pull-in vehicle, and mobile crane assistance. |  |  |
| Plylon |  | 600 |  |  | 630 |  |  | 1000 |  |  |  |  |  |  |  |  | 4 |  |  |  |  | 6 |  |  |
| Plylon |  | 600 |  |  | 1000 |  |  | 1200 |  |  |  |  |  |  |  |  | 4 |  |  |  |  | 6 |  |  |
| Plylon |  | 750 |  |  | 630 |  |  | 1000 |  |  |  |  |  |  |  |  | 4 |  |  |  |  | 6 |  |  |
| Plylon |  | 750 |  |  | 800 |  |  | 1000 |  |  |  |  |  |  |  |  | 4 |  |  |  |  | 6 |  |  |
| Plylon |  | 750 |  |  | 1000 |  |  | 1200 |  |  |  |  |  |  |  |  | 4 |  |  |  |  | 6 |  |  |
| Plylon |  | 900 |  |  | 630 |  |  | 1000 |  |  |  |  |  |  |  |  | 4 |  |  |  |  | 6 |  |  |
| Plylon |  | 900 |  |  | 1250 |  |  | 1200 |  |  |  |  |  |  |  |  | 4 |  |  |  |  | 6 |  |  |
| Plylon |  | 1050 |  |  | 630 |  |  | 1000 |  |  |  |  |  |  |  |  | 4 |  |  |  |  | 6 |  |  |
| Plylon |  | 1050 |  |  | 630 |  |  | 1000 |  |  |  |  |  |  |  |  | 4 |  |  |  |  | 6 |  |  |
| Plylon |  | 1050 |  |  | 800 |  |  | 1000 |  |  |  |  |  |  |  |  | 4 |  |  |  |  | 6 |  |  |
| Plylon |  | 1050 |  |  | 1000 |  |  | 1200 |  |  |  |  |  |  |  |  | 4 |  |  |  |  | 6 |  |  |
| Plylon |  | 1050 |  |  | 1250 |  |  | 1200 |  |  |  |  |  |  |  |  | 4 |  |  |  |  | 6 |  |  |
| Plylon |  | 1050 |  |  | 1600 |  |  | 1200 |  |  |  |  |  |  |  |  | 4 |  |  |  |  | 6 |  |  |
| Plylon |  | 1200 |  |  | 630 |  |  | 1000 |  |  |  |  |  |  |  |  | 4 |  |  |  |  | 6 |  |  |
| Plylon |  | 1200 |  |  | 1600 |  |  | 1200 |  |  |  |  |  |  |  |  | 4 |  |  |  |  | 6 |  |  |
| Plylon |  | 1350 |  |  | 630 |  |  | 1000 |  |  |  |  |  |  |  |  | 6 |  |  |  |  | 8 |  |  |
| Plylon |  | 1350 |  |  | 1250 |  |  | 1200 |  |  |  |  |  |  |  |  | 6 |  |  |  |  | 8 |  |  |
| Plylon |  | 1350 |  |  | 2000 |  |  | 1500 |  |  |  |  |  |  |  |  | 6 |  |  |  |  | 8 |  |  |
| Plylon |  | 1800 |  |  | 2000 |  |  | 1500 |  |  |  |  |  |  |  |  | 6 |  |  |  |  | 8 |  |  |
| Steel Cord |  | 1050 |  |  | ST2500 |  |  | 1500 |  |  |  |  |  |  |  |  | 6 |  |  |  |  | 8 |  |  |
| Steel Cord |  | 1200 |  |  | ST1600 |  |  | 1200 |  |  |  |  |  |  |  |  | 6 |  |  |  |  | 8 |  |  |
| Steel Cord |  | 1500 |  |  | ST1600 |  |  | 1000 |  |  |  |  |  |  |  |  | 8 |  |  |  |  | 10 |  |  |
| Steel Cord |  | 1500 |  |  | ST2000 |  |  | 1200 |  |  |  |  |  |  |  |  | 8 |  |  |  |  | 10 |  |  |
| Steel Cord |  | 1800 |  |  | ST2000 |  |  | 1600 |  |  |  |  |  |  |  |  | 10 |  |  |  |  | 12 |  |  |
| Above schedule excludes additional time to off-load conveyor belt in case of a breakdown. Splice hours will be based on a normal 2 meter pull-in height. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Schedule 3 for Inserts and 2 Splices: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Steel cord and hot splice –Standard inserts of 10 to 20 meter - Time duration per splice per task |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  | Width (mm) |  |  | Class |  |  | Splice Length (mm) |  |  |  |  |  | Total Hours - Splice Only |  |  |  |  |  |  | Total Hours - Insert with 2 x splices –Including clamping and overlap of ends, take-up lifted or de-tensioned, pull-in vehicle, and mobile crane assistance |  |  |  |  |
| Plylon |  | 600 |  |  | 630 |  |  | 1000 |  |  |  |  |  | 6 |  |  |  |  |  |  | 8 |  |  |  |  |
| Plylon |  | 600 |  |  | 1000 |  |  | 1200 |  |  |  |  |  | 6 |  |  |  |  |  |  | 8 |  |  |  |  |
| Plylon |  | 750 |  |  | 630 |  |  | 1000 |  |  |  |  |  | 6 |  |  |  |  |  |  | 8 |  |  |  |  |
| Plylon |  | 750 |  |  | 800 |  |  | 1000 |  |  |  |  |  | 6 |  |  |  |  |  |  | 8 |  |  |  |  |
| Plylon |  | 750 |  |  | 1000 |  |  | 1200 |  |  |  |  |  | 6 |  |  |  |  |  |  | 8 |  |  |  |  |
| Plylon |  | 900 |  |  | 630 |  |  | 1000 |  |  |  |  |  | 6 |  |  |  |  |  |  | 8 |  |  |  |  |
| Plylon |  | 900 |  |  | 1250 |  |  | 1200 |  |  |  |  |  | 6 |  |  |  |  |  |  | 8 |  |  |  |  |
| Plylon |  | 1050 |  |  | 630 |  |  | 1000 |  |  |  |  |  | 6 |  |  |  |  |  |  | 8 |  |  |  |  |
| Plylon |  | 1050 |  |  | 630 |  |  | 1000 |  |  |  |  |  | 6 |  |  |  |  |  |  | 8 |  |  |  |  |
| Plylon |  | 1050 |  |  | 800 |  |  | 1000 |  |  |  |  |  | 6 |  |  |  |  |  |  | 8 |  |  |  |  |
| Plylon |  | 1050 |  |  | 1000 |  |  | 1200 |  |  |  |  |  | 6 |  |  |  |  |  |  | 8 |  |  |  |  |
| Plylon |  | 1050 |  |  | 1250 |  |  | 1200 |  |  |  |  |  | 6 |  |  |  |  |  |  | 8 |  |  |  |  |
| Plylon |  | 1050 |  |  | 1600 |  |  | 1200 |  |  |  |  |  | 6 |  |  |  |  |  |  | 8 |  |  |  |  |
| Plylon |  | 1200 |  |  | 630 |  |  | 1000 |  |  |  |  |  | 6 |  |  |  |  |  |  | 8 |  |  |  |  |
| Plylon |  | 1200 |  |  | 1600 |  |  | 1200 |  |  |  |  |  | 6 |  |  |  |  |  |  | 8 |  |  |  |  |
| Plylon |  | 1350 |  |  | 630 |  |  | 1000 |  |  |  |  |  | 6 |  |  |  |  |  |  | 8 |  |  |  |  |
| Plylon |  | 1350 |  |  | 1250 |  |  | 1200 |  |  |  |  |  | 6 |  |  |  |  |  |  | 8 |  |  |  |  |
| Plylon |  | 1350 |  |  | 2000 |  |  | 1500 |  |  |  |  |  | 6 |  |  |  |  |  |  | 8 |  |  |  |  |
| Plylon |  | 1800 |  |  | 2000 |  |  | 1500 |  |  |  |  |  | 6 |  |  |  |  |  |  | 8 |  |  |  |  |
| Steel Cord |  | 1050 |  |  | ST2500 |  |  | 1500 |  |  |  |  |  | 8 |  |  |  |  |  |  | 10 |  |  |  |  |
| Steel Cord |  | 1200 |  |  | ST1600 |  |  | 1200 |  |  |  |  |  | 8 |  |  |  |  |  |  | 10 |  |  |  |  |
| Steel Cord |  | 1500 |  |  | ST1600 |  |  | 1000 |  |  |  |  |  | 10 |  |  |  |  |  |  | 12 |  |  |  |  |
| Steel Cord |  | 1500 |  |  | ST2000 |  |  | 1200 |  |  |  |  |  | 10 |  |  |  |  |  |  | 12 |  |  |  |  |
| Steel Cord |  | 1800 |  |  | ST2000 |  |  | 1600 |  |  |  |  |  | 12 |  |  |  |  |  |  | 14 |  |  |  |  |
| Above schedule excludes additional time to off load conveyor belt in case of a breakdown. Splice hours will be based on a normal 2 meter pull-in height. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Pulley Lagging Duration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Grip 60 and 70 Diamond pattern lagging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| In Position |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Total Hours - Duration of curing allowance |  |  |  |  |
| Stripping of old rubber |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 Minutes |  |  |  |  |
| Buffing and preparation of metal surface |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 Minutes |  |  |  |  |
| Drying time for metal primer – normal weather conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 Minutes |  |  |  |  |
| Drying time for first coating cement – normal weather conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 15 Minutes |  |  |  |  |
| Second coat of cement and bonding of lagging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 15 Minutes |  |  |  |  |
| Time allowance per pulley |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 Hours |  |  |  |  |
| - Pulley Lagging Duration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Grip 60 and 70 Diamond pattern lagging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Out of Position (workshop) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Duration of curing allowance (Total Hours) |  |  |  |  |
| Stripping of old rubber |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 Minutes |  |  |  |  |
| Buffing and preparation of metal surface |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 Minutes |  |  |  |  |
| Drying time for metal primer – normal weather conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 Minutes |  |  |  |  |
| Drying time for first coating cement – normal weather conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 15 Minutes |  |  |  |  |
| Second coat of cement and bonding of lagging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 15 Minutes |  |  |  |  |
| Time allowance per pulley |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 Hours |  |  |  |  |
| - Pulley Lagging Duration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Grip 60 and 70 Diamond pattern lagging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| In Position |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Duration of curing allowance (Total Hours) |  |  |  |  |
| Stripping of old rubber |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 Hour |  |  |  |  |
| Buffing and preparation of metal surface |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 Minutes |  |  |  |  |
| Drying time for metal primer – normal weather conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 Minutes |  |  |  |  |
| Drying time for first coating cement – normal weather conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 Minutes |  |  |  |  |
| Second coat of cement and bonding of lagging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 15 Minutes |  |  |  |  |
| Time allowance per pulley |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 Hours |  |  |  |  |
| - Pulley Lagging Duration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ceramic lagging as per specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Out of Position (workshop) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Duration of curing allowance (Total Hours) |  |  |  |  |
| Stripping of old rubber |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 Minutes |  |  |  |  |
| Buffing and preparation of metal surface |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 Minutes |  |  |  |  |
| Drying time for metal primer – normal weather conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 Minutes |  |  |  |  |
| Drying time for first coating cement – normal weather conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 15 Minutes |  |  |  |  |
| Second coat of cement and bonding of lagging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 15 Minutes |  |  |  |  |
| Time allowance per pulley |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 Hours |  |  |  |  |

ANNEXURE “B”

SCHEDULE OF RATES FOR ADDITIONAL SERVICES

### Additional Services

#### Rolling of old/redundant Conveyor

The prices of Conveyor Belt rolling or winding or winding is not included in the pricing for replacement or new installations of Conveyor belt prices

#### Conveyor Belt Rolling/Winding

The following rates will be charged for conveyor belt rolling of old conveyors which is pulled and replaced with new conveyor belts. The conveyor belts will be cut in 20m – 30m rolls and strapped ready for removal from the plant.

Rates for belt rolling (by hand):

| Description | Rate | Unit of Measure |
| --- | --- | --- |
| Normal Hours |  | Meter |
| After Hours (1.5) |  | Meter |
| Weekends & Public Holidays (2.0) |  | Meter |

Should the Employer need conveyor rolled in lengths minimum 100m the following rates will be applicable.

Rates for belt winding with belt winding machine:

| Description | Rate | Unit of Measure |
| --- | --- | --- |
| Normal Hours |  | Meter |
| After Hours (1.5) |  | Meter |
| Weekends & Public Holidays (2.0) |  | Meter |

Travelling which includes the transport of the conveyor belt winder will be charged extra, from Kathu Workshop to Khumani Mine, as follows:

| Description | Rate | Unit of Measure |
| --- | --- | --- |
| Normal Hours |  | Trip |

Above mentioned pricing excludes Mobile Crane and suitable machine to pull the belt.

#### The Employer will be responsible for removing the rolls of conveyor belt from site.

ANNEXURE “C”

SITE ESTABLISHMENT COSTS – ONCE OFF

| No. | Description (all PPE as per Khumani standards) | Quantity | Unit Rate (Excluding VAT) | Total Rate (Excluding VAT) | Unit of Measure |
| --- | --- | --- | --- | --- | --- |
| 1 | Medical entry and exit |  |  |  | Each |
| 2 | Registration |  |  |  | Each |
| 3 | Qualification Verification |  |  |  | Each |
| 4 | Screening |  |  |  | Each |
| 5 | ID card (new person) |  |  |  | Each |
| 6 | Training, Class attendance |  |  |  | Each |
| 7 | Licence (Defensive driving) |  |  |  | Each |
| 8 | Safety glasses, clear |  |  |  | Each |
| 9 | Gloves, leather |  |  |  | Pair |
| 10 | Ear Plugs (box of 100) |  |  |  | Each |
| 11 | Locks for machine and equipment lockout |  |  |  | Each |
| 12 | Overall (Company branded, with unique colour) |  |  |  | Pair |
| 13 | Dust masks (box of 100) |  |  |  | Box |
| 14 | Rain Suits |  |  |  | Pair |
| 15 | Hard Hat, normal |  |  |  | Each |
| 16 | Safety Boots, with steel toe |  |  |  | Pair |
| 17 | Freezer Jacket |  |  |  | Each |
| 18 | Freezer Pants |  |  |  | Each |
| 19 | Passport 360 Package, which includes the following: Linked employees, Linked active projects, Own equipment and tools, Online generic SHE files, Reports, Online communication, System/Intranet, Vendor match and Custom dashboard. |  |  |  | Yearly |
| 20 | Salary for Supervisor, responsible for Conveyor Belt Splicing services |  |  |  | Monthly |
| 21 | Salary for Splicer, responsible for Conveyor Belt Splicing services |  |  |  | Monthly |
| 22 | Salary for Semi-Skilled, responsible for Conveyor Belt Splicing services |  |  |  | Monthly |
| 23 | Salary for Skilled Assistant, responsible for Conveyor Belt Splicing services |  |  |  | Monthly |
| 24 | Supply and Installation of Roll Over Protection (ROPS), on Vehicle, UD 6 Ton - Double cab (Toyota/Isuzu) |  |  |  | Each |
| Total Once off site establishment costs (Excluding VAT) |  |  |  |  |  |

ANNEXURE “D”

RATES AND RATES PRICE ADJUSTMENT FORMULA/METHODOLOGY

The Contractor will be entitled to submit a request for adjustment to the rates and prices stipulated in Annexures “A” and “B” to the Contract Agreement, based on the formula contained in this Annexure “D”, 30 (thirty) days prior to 1 November in each year of the Contract.

The Employer will consider the requests for adjustments to the rates and prices, and the parties will be entitled to negotiate such requested adjustments, based on the formula contained in this Annexure “D”.

Should the parties agree the adjustments to the rates and prices during the said 31 day periods between 1 to 31 October annually, the agreed adjustments will be effective on 1 November of each year of the Contract and will be recorded in a formal addendum to the Contract signed by both parties.

Should the parties be unable to agree adjustments to the rates and prices within any of the said 30 day periods, the Employer will be entitled to terminate the Contract on 30 (thirty) days’ written notice furnished to the Contractor.

Tenderer to insert their rates and price adjustment formula/methodology here

PART A.2

## SCOPE OF SITE SERVICES

### Background

#### Assmang (Pty) Limited - Khumani Iron Ore Mine requires Conveyor Belt Splicing Services at Khumani Mine.

### General Description of the Site Services

#### The Scope of Site Services includes the provision for all labour, materials/ consumables and equipment required for the Site Services, which include the following:

##### Hot splices on fabric belting;

##### Hot splices on steel cord conveyor belting;

##### Replacement of Conveyor belting;

##### Insertion of new section in Conveyor belting;

##### Replacement of Conveyor Pulley Lagging;

##### Hot and cold repairs on conveyor belting;

##### Replacement of Conveyor Pulley Lagging Embedded Ceramic Lagging; Including pulling of conveyor belt installation.

##### 1 x Standby team for emergency and or breakdown assistance;

##### Conveyor rolling element maintenance (Conveyor Idlers);

##### Conveyor life expectancy audits.

##### Produce a monthly Conveyor Belt Condition Monitoring Report Indicating:

##### the Condition of splices, Top Cover, Bottom Cover, General Conveyor Belt Condition.

The report is to be submitted to the Engineer / GES/ foremen monthly

### Area of Responsibility and Accountability of Contractor

#### The Contractor will be responsible and accountable for the following activities (Conveyor Belt Service):

##### Hot Splice of fabric belting with vulcanising press only.

##### Hot Splice of Steel cord belting with vulcanising press only. Type of belts on the Sites is as below:

- Bruce Cv10 – 1050mm Steel Cord –Class 2500

- King CV220- 1350 ST 1250 – Plylon belt

- Off Grade CV250 Yard Conv -1200 –ST1600

- On & Off Yard CV250-1200-ST1600

- CV85Off Grade Feed 1800  Steel Cord-2000

- CV40 Off-Grade Yard 1200-1600

- Lab belts, are  400 and 450mm x 2 ply x3 top, 2 bottom

##### Provide – QA/QA: Report on the work done on all repairs. – Hot and Cold including work done on pulleys.   All splices and lagging to be engraved with the date of the splice and the company initials

##### Replacement of new Fabric and Steel cord Belting

##### Installation of Inserts on Fabric and or Steel cord conveyor belt

##### Hot and cold repairs on conveyor belts

##### Replacement of conveyor pulley lagging – In Position

##### Replacement of conveyor pulley lagging - Out of position

##### Replacement of conveyor pulley lagging – Ceramic – In Position

##### Replacement of conveyor pulley lagging – Ceramic – Out of position

##### Conveyor rolling element maintenance i.e. rollers, frames and idler brackets.

##### Conveyor life expectancy audits and reporting.

### Belt Splicing, Replacement, New Installation, Insertion

#### Supply of all belt splicing kits, tools and equipment – belting excluded.

### Supply of generators for splicing.

### Correct Placement of Conveyor Belt rolls prior to work commencement.

#### Correct Stopping of Conveyor Belts prior to replacement, inserting or repairs. (Normal Conditions), and the inspection of take-up devices for detention and position.

#### Securing and recovery of conveyor belts by mean of clamps and mechanical joints under Breakdown conditions.

#### Physical Isolation and De-isolation of equipment at the conveyor drives and substation as per Lock-out Standard.

#### Roll-up and banding – banding strips - of conveyor belting in safe handling lengths.

#### Full face/width Clamping and Cutting of Conveyor belting with all accessories / tools.

#### Removal and Replacement of idler frames / idler rolls for access.

#### Cutting and Rolling of old replaced Conveyor Belt for removal to the salvage yard

#### Good housekeeping after completion of any activities.

### Pulley Lagging

#### Removal of old lagging

#### Installation of pulley lagging

#### The Contractor will provide all lagging material and tools needed for pulley lagging:

#### Shore degree Grip 60 x 10mm Diamond pattern lagging for pulleys up to face width of 900 mm

#### Shore degree Grip 70 x 12mm Diamond pattern lagging for pulleys wider than 900mm face.

#### Perform – sponge blasting On all Conveyor – Drive, Tail and Head Pulleys.

#### Recommend technical specification for each of the following critical applications: Drive, Tail and Head Pulley Lagging.

#### Embedded ceramic lagging – Type  Nilos, strip lagging, 12mm thick, 4 piece diamond pattern, bonding layer, rubber shore hardness = 50+5, tiles – min. of 92% Aluminium Oxide with minimum tensile strength of 20Mpa.

### Hot and Cold Repairs on Conveyor Belts

#### Supply of all repair cover strips tools and equipment as defined per work order.

#### Correct Stopping of Conveyor Belts prior to repair.

#### Physical Isolation and De-isolation of equipment at the conveyor drives and substation as per Lock-out Standard.

#### Removal and Replacement of idler frames / idler rolls for access.

#### Good housekeeping after completion of any activities.

### Conveyor Rolling Element Maintenance – 5 Days per Week

#### Supply labour, tools and equipment for conveyor belt maintenance on idler roller replacement.

#### Idler, idler frame and bracket replacement.

#### Continuous improvement suggestions/proposals.

### Conveyor Life Expectancy Audit and Reporting – Quarterly

#### Supply of labour tools and equipment for conducting life expectancy audits on critical conveyor belts.

#### This Inspection must include – Inspecting Conveyor Belt rolls in the Stores.

#### Auditing of physical fabric belt condition and life expectancy.

#### Auditing of physical steel cord belt condition and life expectancy.

#### Quarterly Scanning of Steel belt cord and deviation reporting.

#### Continuous improvement suggestions.

### Standby Team

#### Provide for 1 x standby team for emergency and or breakdown maintenance.

#### Standby team to be available on a 7 day working week, 365 days per annum.

#### Maximum reaction time of +/- 2 hours from call-out to site.

### Additional Responsibility and Accountability for the Contractor

#### Additional work teams may be required during a scheduled shutdown whereupon the Contractor must commit himself (in writing) to complete the work required.

#### Completion and acquiring of necessary Employer’s signatories for invoicing.

#### Complete and submit work orders to the responsible Employer’s Supervisor

#### Completion and sign–off of Employer’s Work orders.

#### Deliver Proof of Training to operate / use belting equipment as well as Competency Training to perform duties associated with conveyor services.

#### Proof of risk assessments to be submitted to the mine representative prior to commencing any activities.

#### The Contractor to supply Personnel Protective Equipment to all employees.

#### The Contractor to submit all hazardous Materials Safety Data Sheets (MSDS) of hazardous materials to be used and to ensure that his personnel adhere to these requirements.

#### The Contractor will be responsible for all transportation – to and from site, on site and all vehicles will comply with the minimum standards of Khumani Iron Ore Mine.

#### Should the Contractor have to wait before it can perform any of the Site Services in terms of the Contract, for reasons attributable to the Employer, such waiting time will be charged at the rates stipulated in Annexure “A” to the Contract Agreement, in the table titled “Waiting Time”.

### Area of Responsibility and Accountability for the Employer:

#### Belt Splicing / Pull-In / Repairs

##### Fabric and or steel cord belting supply.

##### Mobile crane.

##### Pull-in vehicle or machine.

##### Lift / lower of take-ups and lifting equipment.

##### Off-loading of belt in case of an emergency.

##### Removal of tension and tensioning conveyors on screw take-ups.

##### Removal and replacement of nip guards and handrails.

##### Signing of relevant documents.

##### Training belt upon completion of work.

#### Pulley Lagging: (In Position)

##### Mobile crane.

##### Scaffold erection and dismantling.

##### Lift / lower of take-ups and lifting equipment.

##### Removal of tension and tensioning on screw take-ups.

##### Remove / install of nip guards.

##### Remove / install coupling pins / connections.

##### Training of belt upon completion of work.

#### Pulley Lagging: (Out of Position)

##### Power supply for equipment.

##### Mobile crane.

##### Lift / lower of take-ups and lifting equipment.

##### Removal of tension and tensioning on screw take-ups.

##### Remove / install of nip guards.

##### Remove / install coupling pins / connections.

##### Remove / install pulley.

##### Transport to workshop.

##### Training of belt upon completion of work.

### Hot And Cold Repairs On Conveyor Belting

##### Power supply for tools

##### Off-loading of belt in case of an emergency

##### Removal and Replacement of nip guards and handrails

##### Signing of relevant documents.

#### Conveyor Rolling Element Maintenance

##### Supply of all idler as per request.

##### Supply of all idler frames as per request.

##### Provide maintenance data for continuous improvement.

#### Conveyor Life Expectancy Audit And Reporting

##### Scheduled windows for auditing.

##### Scheduled windows for steel cord scanning.

##### Maintenance data for continuous improvement.

#### Warranty

Provide Standard warranty times as follows:

##### Pulley Lagging - For rubber to metal Bonding

##### Plylon - Cold / Hot splices – New belt

##### Plylon - Cold / hot splices – aged belt

##### Steel cord splices Matador belt – new

##### Steel cord splices other belt – new

##### Steel cord splices other belt – aged

The Contractor only offers a guarantee if the condition of the belt and the working conditions allow for it.

Warranties are given on a “pro rata temporis“ basis and can only be honoured it when notified without delay.  For the avoidance of doubt, if a splice lasts for 50% of its warranted life expectancy, the Contractor will redo the splice and will bear 50% of the cost thereof, with the Employer placing an order for the remaining 50% cost. The new splice will than carry a full warranty.

### General Conditions

#### The Employer can require more than 1 team per day to execute scheduled or breakdown work.

#### The Employer cannot guarantee full time employment of Contractor labour and work will be executed on an “as and when request”.

#### Noncompliance and substandard work can lead to Contract termination.

#### The Employer will be entitled to appoint more than one service provider for work execution during extended maintenance windows and/or breakdowns.

#### The Employer will not supply normal tools, or equipment to the Contractor, i.e. oxygen, acetylene, welding rods, impact wrenches, sockets, welding machines, belting tools, basics slings, shackles clamps or slings, ext.

#### Only special tools, being steel rope, 16mm, 50m, for pulling in of conveyor belts, and steel rope slings or chain slings with a capacity of more than 3000kg, will be supplied to the Contractor when required.

#### All Contractor employees must be fully qualified, trained and competent in their field of work.

#### The Contractor must adhere to the Standard lockout procedure.

#### The Contractor must adhere to the Employer’s permit to work system, including additional permits that need to be required.

#### The lockout register must be completed in the substation as per standard. All padlocks will be clearly marked for identification purposes. Only for the stackers and reclaimers.

#### The contractor will ensure that all CTA, PTO and special procedures be updated

#### to mine standards

#### The Contractor will not be allowed to weld or flame cut on any structures or make any safety device ineffective.

#### All Contractor vehicles, equipment and tools will be tested and inspected before commencing with the execution of the Contract.

#### The Contractor must assist the Employer in cases of emergency/breakdown work during weekends and after hours.

#### All Contractor tools and equipment must be recorded in a log book and inspected on a monthly basis.

#### The Contractor must ensure overtime legal compliance; employees exceeding the 40 hour overtime limit must be replaced with additional labour.

#### The Contractor must provide all splicing and/or pulley lagging procedures within 14 (fourteen) days of the Commencement Date of the Contract.

### Service Conditions

#### Prices are based on the assumption that the working conditions and the work area are suitable for superior workmanship and that there is easy access for materials and equipment.

#### The Employer is responsible for power of 220v-30amps.min and 380v-80amps.min within 20 m from the work area.

#### For 24-hour service work the following labour rate categories are applicable:

Mon-Fri	07:00 - 16:00	- Normal Hours rates

Mon-Fri	16:00 – 07:00 - After Hours rates

Fri-Sat	- 16:00 - 7:00	- After Hours rates

Sat-Mon - 7:00 - 7:00	- Weekends / Public Holiday Rates

Note: Should work be carried out in different time zones, the rates which will apply will be the rates applicable to the time zone in which the majority of the work has been carried out.

#### All Transport to, from and on the mine is the responsibility of the Contractor.

#### Any extra travelling from Kathu Workshop to Khumani Mine and back, which is not included in the Schedule of Rates and Prices for Conveyor Splicing and Pulley Lagging, will be charged extra at R per trip.

Note:

The above travelling price can change due to statutory fuel price fluctuations

### Continuous Improvement

#### The Contractor will identify 1 (one) continuous improvement initiative per year.

#### The Contractor will present these initiatives to the Employer’s 3 Senior Engineers responsible for King, Bruce, Parsons and Load-out, as well as Supply Chain Management to obtain written approval before implementing any initiatives.

### Administrative Controls

#### The Employer’s Supervisor, General Engineering Supervisor and Engineer will sign off the invoices on a monthly basis.  The Contractor will attach the clocking sheets of its employees together with the signed job cards to the monthly invoice. The Employer’s Supervisor, General Engineering Supervisor and Engineer will review the clocking sheets and the job cards for accuracy before signing the monthly invoice/s.

### Key Performance Indicators

#### The performance of the Contractor will be measured on a monthly basis against the following Key Performance Indicators (KPI’s):

##### Safety

| KPI No. | KPI | TARGET | Tolerance |
| --- | --- | --- | --- |
| 1 | Safety: LTI’s | 0 | Zero tolerance |
| 2 | Safety: Mini risk assessments | 100% | Zero tolerance |
| 3 | Adherence to Inspection schedule | 100% | Zero tolerance |
| 4 | Compliance to MHSA | 100% | Zero tolerance |

##### Operational

| KPI No. | KPI | Target |
| --- | --- | --- |
| 5 | Compliance to quality of service | 100% Compliance |
| 6 | Services in accordance with Contract scope of work | 90% Compliance to scope of work |
| 7 | Breakdown / Turnaround time Conveyor Belt Splicing Maintenance crew availability | Response < 2 hours on breakdowns |
| 8 | Implement continuous improvement initiatives to assist in reducing Employer’s operating costs and to minimise downtime | 1 x continuous improvement initiative per year |
| 9 | Completion and submission of job cards | 100% |
| 10 | Housekeeping | Checklist with score will be completed by Engineering Supervisors |

#### The targets will be adjusted annually based on the Employer’s requirements.

#### In the event that the Contractor fails to comply with the Safety KPI’s 1 – 4 above, the Employer will be entitled to cancel the Contract summarily, upon written notice to this effect.

#### In the event that the Contractor fails to comply with any of the Operational KPI targets 5 – 7, 9 and 10 above, the following process will be followed:

##### All monthly KPI’s not achieved for one month, the non-performing KPI’s will be discussed at the quarterly meeting, or alternative meeting arranged if need;

##### KPI’s not achieved for any two (2) months of the Employer’s financial year, a Non-conformance letter will be issued to the Contractor.  The Contractor must present action plan to address Non-conformance;

##### KPI’s not achieved for any three (3) months of the Employer’s financial year, a breach notice will be issued to the Contractor. Contractor to remedy breach within 7 (seven) days;

##### All KPI’s not achieved for any four (4) months of the Employer’s financial year, the Employer will be entitled to cancel the Contract summarily, upon written notice to this effect.

#### For the purpose of this Sub-Clause, the Employer’s financial year is 01 July to 30 June.

### Contractor Management Meetings

#### A monthly Contract management meeting will be held on-Site at Assmang Khumani, or alternatively via the Microsoft Teams platform with key Engineering and Supply Chain Management stakeholders from the Employer and the Contractor. This meeting will discuss:

##### Occupational health and safety.

##### Performance and KPI’s.

##### Service delivery claims and quality issues.

##### Continuous improvement initiatives and implementation.

##### Recommendations from the Contractor.

##### Any other Contract management performance and/or management issues identified during the preceding quarter.

### Communications and Reporting

#### See Annexure A1 for minimum communications and reporting requirements (excluding Notices in terms of the Contract).

### Safety, Health and Environment

#### See Annexure A2 for minimum safety, health and environmental requirements.

### Quality Management

#### See Annexure A3 for minimum quality management requirements.

### Regulations, Codes, Standards and Specifications

#### See Annexure A4 for details on the regulations, codes, standards and specifications applicable to the Contractor in supply the Commodities.

### Attachments

#### Attachment 1	-	Standards, Procedures and Specifications

ANNEXURE A1: COMMUNICATIONS AND REPORTING

| ANNEXURE No. A1 |  |  |
| --- | --- | --- |
| TITLE: Communications and Reporting (other than Notices given in terms of the Contract) |  |  |
|  |  | - Communication will be done via e-mail or letter with proof of receipt on an ongoing basis, and in accordance with the provisions of the Contract. - The Contractor shall submit a written monthly progress report to the three Senior Engineers of Parsons, Bruce & King Site by the 7th of each month, covering the period from beginning to the end of the preceding calendar month and highlighting the following: - Activities completed during preceding reporting period per Belt Splicing Maintenance Team; - Activities in progress during current reporting period per Belt Splicing Maintenance Team; - Activities to be undertaken during next reporting period per discipline; - Areas of concern; - Updated or revised programme and progress details as described above; - Recovery plan (when applicable); - Man-hour progress including original budget vs actual vs forecast man hours and cost; - - Lists of deviations / variations and - Actions taken or to be taken to minimise or negate changes. |

ANNEXURE A2: SAFETY, HEALTH AND ENVIRONMENTAL

| ANNEXURE No. A2 |  |  |
| --- | --- | --- |
| TITLE: Safety, Health and Environmental |  |  |
|  | 1. | The Contractor shall provide the necessary safety, health and environmental (SHE) management system in accordance with the Mine Health and Safety Act. |
|  | 2. | The Contractor shall provide all PPE pertaining to the Site Services. |
|  | 3. | The Contractor shall comply with the Employer’s SHE work rules and regulations listed in the table below. The Contractor acknowledges receipt of an electronic copy of these documents and agrees to be bound by what is contained therein. |

ANNEXURE A3: QUALITY MANAGEMENT

| ANNEXURE No. A3 |  |  |  |
| --- | --- | --- | --- |
| TITLE: Quality Management |  |  |  |
|  | 1. |  | The Contractor shall monitor its activities in accordance with his approved Contract quality plan and notify the Employer of any deviations to this plan. The Contractor shall rectify any deviations from the Contract quality plan with subsequent approval by the Employer. |  |

ANNEXURE A4: REGULATIONS, STANDARDS AND SPECIFICATIONS

| ANNEXURE No. A4 |  |  |  |
| --- | --- | --- | --- |
| TITLE: Regulations, Codes, Standards and Specifications |  |  |  |
|  | 1. |  | The Contractor shall comply strictly with all National Acts, Provincial Ordinances and Acts, Municipal Bye-laws, and any other laws, regulations, directions, permissions and other requirements which are applicable to the Contractor’s supply as described in the Scope of Supply, as amended from time to time, including, but without being limited to the: - Mine Health and Safety Act (29/1996) and the Regulations thereto - Compensation for Occupational Injuries and Diseases Act (130/1993) - Occupational Diseases in Mines and Works Act and Regulations (78/1973) - Unemployment Insurance Act (30/1966) - Health Act and Regulations (63/1977) - Labour Relations Act and Regulations (66/1995) - Mines and Works Act (27/1956) (providing for Sunday work) - Occupational Health and Safety Act and Regulations (85/1993) - Basic Conditions of Employment Act (75/1997) |
|  | 2. |  | All materials supplied shall meet or exceed the applicable South African legislation and standards. Where no standards are applicable, generally accepted procedures for supply, if applicable, will be observed. |
|  | 3. |  | The Contractor shall refer to the attached Employer’s specifications and as far as possible adhere to these specifications. Where the standards in this section significantly conflict with the standard methodologies used by the Contractor, or significantly increase costs or risks, the Contractor shall advise the Employer. |
|  | 4. |  | COMPLIANCE TO MHSA Prospective Service Provider: to Provide a plan on how will compliance be achieved for : Regulation 8.9(1) to 8.9(9) |
|  | 5. |  | Any proposed deviation from the specification, including modifications deemed necessary in particular applications, shall be submitted to the Employer for approval, prior to implementation. The Employer’s approval shall not absolve the Contractor’s responsibility in terms of the design and supplying the Commodities. |

ATTACHMENT 1: STANDARDS, PROCEDURES AND SPECIFICATIONS

The Contractor acknowledges receipt of an electronic copy of these documents and agrees to be bound by what is contained therein.

| DOCUMENT NO. | REV |  | TITLE |
| --- | --- | --- | --- |
| EMPLOYER’S STANDARDS, PROCEDURES AND SPECIFICATIONS |  |  |  |
| SP_160949 | 2.0 | Khumani Contractor Management Procedure |  |
| S_E77654 | 1.0 | Khumani Mine Traffic Management Standard |  |
| COP_E77601 | 1.0 | Mandatory Code Of Practice For Trackless Mobile Machines |  |
| OF_971523 | 1.0 | TMM To Site Acceptance Form |  |
| 1.2.1 Strategies and Policies | 2.0 | Khumani Mine Golden Rules |  |
| SP_675757 | 5.0 | Khumani Mine SHERQ Risk Assessment and Management Procedure |  |
| SP_976288 | 5.0 | Engineering Logbook Standard Procedure |  |
| SP_ 175332 | 3.0 | PPE Procedure |  |
| COP_T76463 | 2.0 | Khumani Mine Mandatory Emergency Preparedness And Response COP |  |
| COP_375959 | 1.0 | Mandatory Code Of Practice For An Occupational Health Programme For Noise |  |
| PRO-ES-44 | 6.0 | Khumani Mine Waste Management Procedure |  |
| S_T74945 | 2.0 | Khumani mine Operations Fatigue Standard |  |
| COP_375795 | 2.0 | Occupational Health Programme on Thermal stress |  |
| S_E74168 | 3.0 | Khumani Mine Lock-out Standard |  |
| P_E76792 | 1.0 | Khumani Working at Heights Procedure |  |
| SP_972649 | 1.0 | Khumani Integrated Traffic Management |  |
| COP_359072 | 1.0 | Khumani Trackless Mobile Machine Parking Procedure |  |
| SP_876607 | 1.0 | Quality Assurance Management Procedure |  |
| OF_175736 | 1.0 | SHERQ Health And Safety Representative Inspection List |  |
| OF_168403 | 1.0 | Incident Investigation On Property Damages And Non-Conformances |  |
| COP_T77069 | 1.0 | Khumani Mine Mandatory on Minimum Standards Of Fitness To Perform Work At a Mine |  |

| APPROVAL - PURCHASER |  |  |  |
| --- | --- | --- | --- |
| RESPONSIBLE PERSON | FUNCTION | SIGNATURE | DATE |
| G CLARK | Originator – Draft Scope of Work |  |  |
| GES Jigs and Load out |  |  |  |
| A TSHINEVA | Review Scope of Work & insert applicable Engineering information |  |  |
| Engineer Parsons |  |  |  |
| ITUMELENG TSHALE | Review Scope of Work & insert applicable Engineering information |  |  |
| Engineer JIGS and Load-out |  |  |  |
| PONTSHO LESETEDI | Review Scope of Work & insert applicable Engineering information |  |  |
| Engineer King |  |  |  |
| F MPOFANA | Review & Recommend Scope of Work |  |  |
| Senior Engineer Parsons and Load-out |  |  |  |
| FRANCOIS BEZUIDENHOUT | Review & Recommend Scope of Work |  |  |
| Senior Engineer Bruce |  |  |  |
| MICHAEL CHEESIAN | Review & Recommend Scope of Work |  |  |
| Senior Engineer King |  |  |  |
| DEON VICTOR | Review Scope of Work & insert applicable Safety & Risk related information |  |  |
| Manager Safety & Risk |  |  |  |
| KENRIC BARBERY | Review Scope of Work & insert applicable Training related information |  |  |
| Manager Training |  |  |  |
| DIRK COETZEE | Review Scope of Work & insert applicable Environmental information |  |  |
| Superintendent Environmental |  |  |  |
| NANDI MABUZA | Review Scope of Work & insert applicable Hygiene information |  |  |
| Superintendent Hygiene |  |  |  |
| ASHLEY ENGELBRECHT | Review Scope of Work & insert applicable Contractor Management information |  |  |
| Superintendent Contractor Management |  |  |  |
| KAY MULLER | Review Scope of Work & insert applicable HR related information |  |  |
| Manager HR Operations |  |  |  |
| JANEREZ ELOFF | Review Scope of Work & insert applicable Finance related information |  |  |
| Manager Financial Services |  |  |  |
| Pieter | Review & approve Scope of Work |  |  |
| Manager HOD Engineering |  |  |  |